DAT-WATER-PLC-001

Domestic Water Replenishment Controller

Cape Town House — Technical Specification

Document Control

| Field | Value |
| --- | --- |
| Document ID | DAT-WATER-PLC-001 |
| Title | Domestic Water Replenishment Controller — Cape Town House |
| Version | v0.3 |
| Date | 2026-07-18 |
| Classification | Internal — Technical |
| Prepared by | Misha (AI Agent) |
| Reviewed by | [blank — pending Zun review] |
| Approved by | [blank — pending Zun approval] |
| Supersedes | DAT-WATER-PLC-001 v0.2-Misha (2026-07-18) |
| Distribution | Zun (owner), Project record |
| Retention period | 5 years from project completion |
| Document control | Subject to change control. Hand-written annotations are not permitted. |

Revision History

| Version | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 0.1 | 2026-07-16 | Previous Agent | Initial draft (incorrect topology — cistern as ground storage) |
| 0.2 | 2026-07-18 | Misha | Topology corrected to toilet cistern; BDD pump; previous-agent references removed; 4 external drawings |
| 0.3 | 2026-07-18 | Misha | Rabtron Wahwang LED datasheet-verified BOM; ISO 9001 control block; ESP32-C6 GPIO map v2; HA integration; ESPHome YAML v2 |

1. Scope and Purpose

This specification defines the design, build, and commissioning requirements for a domestic water replenishment controller (Water PLC) installed at a 1970 Cape Town house. The system automatically replenishes a toilet cistern overnight using water from an existing borehole via a JoJo tank storage system.

The system is classified as extra-low voltage (ELV) per SANS 10142-1. No ICASA or NRCS type approval is required for this installation.

2. System Architecture

2.1 Topology

The system uses a stacked-tank architecture:

Borehole well point (existing) → existing 220V AC well pump (existing, 0.75 kW)

Well pump → JoJo tank (ground level, ~1000L black plastic)

JoJo tank → BDD 12V submersible pump (external, gravity-fed from tank bottom outlet)

BDD pump → toilet cistern (wall-mounted, ~11–12L black plastic, ~1.5m above floor)

Cistern float switch signals demand to ESP32-C6 controller

2.2 Block Diagram

Mains 220V AC → Pool Sub-DB (10A MCB) → Mornsun LM50-23B12R2S PSU → 12V busbar → LM2596 Buck → 5V rail → ESP32-C6 → BDD Pump + Aviation LEDs + Toilet RGB + Sensors

2.3 Controller

ESP32-C6 DevKitC-1 (N16R8, 8MB flash, 19 GPIO, WiFi 6, BLE 5). All logic runs on ESPHome firmware.

3. Bill of Materials (Engineering Grade)

| Item | Description | Mfr PN | Supplier PN | Supplier | Qty | Unit ZAR | Total ZAR | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P-01 | Well pump (EXISTING) | — | — | — | 1 | — | — | Existing |
| P-02 | BDD 12V Solar Heat Water Pump | BK-12V | 3222-BK | Communica | 1 | 250.00 | 250.00 | In stock |
| E-01 | ESP32-C6 DevKitC-1 (N16R8) | ESP32-C6-DevKitC-1-N16R8 | ESP32-C6-DEV | Micro Robotics | 1 | 195.00 | 195.00 | In stock |
| E-02 | Mornsun PSU 12V/4.2A 50W | LM50-23B12R2S | LM50-23B12R2S | Communica | 1 | 265.00 | 265.00 | In stock |
| E-03 | DC-DC Buck Module LM2596S (Zun has) | LM2596S | — | Zun has | 1 | 0.00 | 0.00 | Have |
| E-04 | DS3231 RTC Module (ZS-042) | ZS-042 | ZS-042 | Communica | 1 | 85.00 | 85.00 | In stock |
| E-05 | OLED Display 0.96" 128×64 I²C | SSD1306 | SSD1306-096 | Communica | 1 | 65.00 | 65.00 | In stock |
| E-06 | HL-52S Relay Module (5V coil) | HL-52S | HL-52S | Communica | 1 | 55.00 | 55.00 | In stock |
| E-07 | AO3400A N-Channel MOSFET | AO3400A | AO3400A | Communica | 1 | 5.00 | 5.00 | In stock |
| E-08 | A02YYUW Ultrasonic Level Sensor | A02YYUW | A02YYUW | Communica | 1 | 145.00 | 145.00 | In stock |
| E-09 | YF-S201 Hall-Effect Flow Meter | YF-S201 | YF-S201 | Communica | 1 | 95.00 | 95.00 | In stock |
| E-10 | Cistern Float Switch (reed) | Float-PP | FLOAT-PP | Communica | 1 | 35.00 | 35.00 | In stock |
| L-01 | 10mm White Super Bright LED | WW10A3SWQ4-N2 | #2824 | Rabtron | 1 | 10.00 | 10.00 | In stock 22065 |
| L-02 | 10mm Red Super Bright LED | WW10A3SRQ4-N2 | #3224 | Rabtron | 1 | 10.00 | 10.00 | In stock |
| L-03 | 10mm Blue Super Bright LED | WW10A3SBQ4-N2 | #3133 | Rabtron | 1 | 10.00 | 10.00 | In stock |
| L-04 | 10mm Orange Super Bright LED | WW10A3OYF4-N2 | #3222 | Rabtron | 1 | 10.00 | 10.00 | In stock |
| L-05 | 3mm WS2812B RGB LED | WS2812B | WS2812B-3MM | Communica | 1 | 8.00 | 8.00 | In stock |
| R-01 | Resistor 220Ω 1/4W 5% (×2 Red/Orange) | 220R-5% | — | Rabtron | 2 | 0.50 | 1.00 | In stock |
| R-02 | Resistor 120Ω 1/4W 5% (×2 White/Blue) | 120R-5% | — | Rabtron | 2 | 0.50 | 1.00 | In stock |
| R-03 | Resistor 10kΩ 1/4W 5% (×8 pull-ups) | 10KR-5% | — | Rabtron | 8 | 0.50 | 4.00 | In stock |
| R-04 | Resistor 4.7kΩ 1/4W 5% (×2 I²C) | 4K7-5% | — | Rabtron | 2 | 0.50 | 1.00 | In stock |
| C-01 | Capacitor 100µF 25V Electrolytic | 100UF-25V | — | Rabtron | 2 | 2.00 | 4.00 | In stock |
| C-02 | Capacitor 100nF Ceramic | 100NF-50V | — | Rabtron | 5 | 1.00 | 5.00 | In stock |
| TB-01 | Terminal Block 2-way 5mm pitch | TB-2W-5MM | TB-2W-5MM | Communica | 3 | 8.00 | 24.00 | In stock |
| MCB-01 | MCB 10A Single-Pole C-Curve 6kA | S261C10 | S261C10 | Communica | 1 | 125.00 | 125.00 | In stock |
| MCB-02 | MCB 6A Single-Pole C-Curve 6kA | S261C06 | S261C06 | Communica | 1 | 125.00 | 125.00 | In stock |
| MCB-03 | MCB 3A Single-Pole C-Curve 6kA | S261C03 | S261C03 | Communica | 1 | 125.00 | 125.00 | In stock |
| MCB-04 | DC MCB 2A Single-Pole DC-rated (CRITICAL) | DC2A-C | DC2A-C | Communica | 1 | 185.00 | 185.00 | In stock |
| CB-01 | 4mm² 3-core TPS cable | TPS-4MM2-3C | TPS-4MM2-3C | Communica | 5 | 35.00 | 175.00 | In stock |
| CB-02 | 1.5mm² 2-core flex cable | FLEX-1MM2-2C | FLEX-1MM2-2C | Communica | 10 | 15.00 | 150.00 | In stock |
| CB-03 | 1.5mm² 4-core signal cable | SIG-1MM2-4C | SIG-1MM2-4C | Communica | 5 | 25.00 | 125.00 | In stock |
| CT-01 | 20mm PVC conduit UV-stable | PVC-20MM | PVC-20MM | Communica | 4 | 18.00 | 72.00 | In stock |
| ENC-01 | Hammond RP1465C ABS Enclosure IP65 | RP1465C | RP1465C | Communica | 1 | 245.00 | 245.00 | In stock |
| DR-01 | DIN Rail TS35 slotted 215mm | DR-TS35-215 | DR-TS35-215 | Communica | 1 | 28.00 | 28.00 | In stock |
| DR-02 | DIN Rail Panel Mount Adapter | DR-ADAPT | DR-ADAPT | Communica | 1 | 18.00 | 18.00 | In stock |
| GL-01 | PG9 Cable Gland IP68 | PG9-IP68 | PG9-IP68 | Communica | 2 | 12.00 | 24.00 | In stock |
| GL-02 | PG7 Cable Gland IP68 | PG7-IP68 | PG7-IP68 | Communica | 4 | 8.00 | 32.00 | In stock |
| EB-01 | Earth Bar Brass 6-way DIN-rail | EB-6W-DR | EB-6W-DR | Communica | 1 | 45.00 | 45.00 | In stock |
| SP-01 | Earth Spike 1.2m galvanised | ES-1M2-G | — | Local hardware | 1 | 95.00 | 95.00 | Local |
| PL-01 | 1/2" to 15mm HDPE adapter | AD-15MM-12 | — | Local plumber | 2 | 25.00 | 50.00 | Local |
| PL-02 | 15mm HDPE pipe (roll) | HDPE-15MM | — | Local hardware | 20 | 8.00 | 160.00 | Local |
| PL-03 | Anti-vibration mount (pump) | AVM-PUMP | AVM-PUMP | Communica | 4 | 15.00 | 60.00 | In stock |
| SW-01 | 3-position toggle switch OFF/AUTO/MAN | SW-3P-MTU | SW-3P-MTU | Communica | 2 | 35.00 | 70.00 | In stock |
| SW-02 | Momentary push-button NO M12 | BTN-NO-M12 | BTN-NO-M12 | Communica | 3 | 18.00 | 54.00 | In stock |
| LED-HS-01 | M12 Metal LED Bezel Housing | M12-LED-BZ | M12-LED-BZ | FastenRight | 4 | 22.00 | 88.00 | In stock |
| FT-01 | Teflon tape (plumbing seal) | PTFE-TAPE | — | Local hardware | 1 | 15.00 | 15.00 | Local |
| HS-01 | Heatshrink 3mm black (5m roll) | HS-3MM-BK | HS-3MM-BK | Communica | 1 | 25.00 | 25.00 | In stock |
| CN-01 | Cable markers (printable, 50 pcs) | CABLE-MARK | CABLE-MARK | Communica | 1 | 35.00 | 35.00 | In stock |

TOTAL ESTIMATED COST: ~R3,000 ZAR (excl. existing equipment, labour, conduit, plumbing fittings)

4. Controller Specification

4.1 ESP32-C6 Pin Assignment (v3)

| GPIO | Type | Function | Component | Interface | Notes |
| --- | --- | --- | --- | --- | --- |
| GPIO2 | DI | Cistern float switch | Reed switch | Input, pull-up 10kΩ | Dry contact, active LOW |
| GPIO3 | DI | Flow meter pulse | YF-S201 | Interrupt input | F=7.5×Q Hz, 10kΩ pull-up |
| GPIO4 | DI | Well pump mode switch | 3-position toggle | Input, pull-up 10kΩ | OFF/AUTO/MANUAL |
| GPIO5 | DI | JoJo pump manual button | Momentary NO | Input, pull-up 10kΩ | Manual override |
| GPIO6 | IO | I²C SDA | OLED + DS3231 RTC | I²C bus | 4.7kΩ pull-ups |
| GPIO7 | IO | I²C SCL | OLED + DS3231 RTC | I²C bus | 4.7kΩ pull-ups |
| GPIO10 | DO | Aviation LED Red | Wahwang WW10A3SRQ4-N2 | LEDC PWM 1kHz | 220Ω resistor, 5V rail |
| GPIO11 | DO | Aviation LED White | Wahwang WW10A3SWQ4-N2 | LEDC PWM 1kHz | 120Ω resistor, 5V rail |
| GPIO12 | DO | Aviation LED Blue | Wahwang WW10A3SBQ4-N2 | LEDC PWM 1kHz | 120Ω resistor, 5V rail |
| GPIO13 | DO | Aviation LED Orange | Wahwang WW10A3OYF4-N2 | LEDC PWM 1kHz | 220Ω resistor, 5V rail |
| GPIO15 | DO | BDD pump MOSFET gate | AO3400A | Digital out | 10kΩ pull-down, strap pin (safe with pull-down) |
| GPIO16 | DO | Toilet RGB data | WS2812B 3mm | NeoPixel bus | Single-wire RGB control |
| GPIO18 | DO | Well pump relay coil | HL-52S module | Digital out | Opto-isolated, 5V coil |
| GPIO20 | IO | A02YYUW UART RX | Sensor TX (yellow wire) | UART2, 9600 baud | 3-450 cm range, IP67 |
| GPIO21 | IO | A02YYUW UART TX | Sensor RX (white wire) | UART2, 9600 baud | 3-450 cm range, IP67 |

Spare GPIOs: GPIO0, GPIO1, GPIO8, GPIO9, GPIO19 (5 available for future expansion).

4.2 Power Rails

12V rail: From Mornsun LM50-23B12R2S, drives BDD pump, MCB-04, well pump relay coil

5V rail: From LM2596 buck (set to 5.0V ±0.1V), drives ESP32, WS2812B toilet LED, aviation LEDs

3.3V rail: From ESP32-C6 on-board LDO, drives I²C pull-ups, A02YYUW (via ESP32), YF-S201

4.3 Time Schedule

Normal refilling window: 17:00 to 07:00 daily (14-hour window)

Schedule enforced via NTP-synchronised system clock

DS3231 RTC maintains time during WiFi outage

Manual override bypasses schedule

5. External Drawing Pack

The following drawings are provided as separate A3-format HD printable files (300 DPI, colour):

| Drawing | Title | Format | Scale | SVG File | PNG File |
| --- | --- | --- | --- | --- | --- |
| D-01 | Piping and Instrumentation Diagram (P&ID) | SVG + 300 DPI PNG | 1:50 | D-01-P-and-ID-v0.3.svg | D-01-P-and-ID-v0.3.png |
| D-02 | Electrical Single-Line Diagram | SVG + 300 DPI PNG | 1:20 | D-02-Electrical-Single-Line-v0.3.svg | D-02-Electrical-Single-Line-v0.3.png |
| D-03 | ESP32-C6 IO Wiring Diagram | SVG + 300 DPI PNG | 1:1 | D-03-ESP32-C6-IO-Wiring-v0.3.svg | D-03-ESP32-C6-IO-Wiring-v0.3.png |
| D-04 | Enclosure Layout (Hammond RP1465C) | SVG + 300 DPI PNG | 1:2 | D-04-Enclosure-Layout-v0.3.svg | D-04-Enclosure-Layout-v0.3.png |

All drawings: A3 landscape, 300 DPI, colour, print-ready. Print all 4 drawings and staple with this specification.

6. Water Savings Calculation

6.1 Baseline

4-person household (2 adults, 2 children). Toilet cistern flush volume: 11L (SA standard single-flush, SANS 5054).

6.2 Calculation

Flushes per person per day: 5 (per WaterWise SA guidelines)

Total flushes per day: 4 × 5 = 20

Water saved per flush: 11L (municipal supply displaced by borehole)

Daily saving: 20 × 11L = 220 L/day

Monthly saving: 220 × 30 = 6,600 L = 6.6 kL/month

Annual saving: 220 × 365 = 80,300 L = 80.3 kL/year

6.3 Cost Benefit (Cape Town 2025/26 Tariff)

Using City of Cape Town Step 1 residential tariff (R24.32/kL for 0-6 kL/month):

Monthly saving: 6.6 kL × R24.32/kL = R160.51/month

Annual saving: R160.51 × 12 = R1,926/year

At higher consumption tiers the saving is proportionally higher (Step 2: R32.97/kL = R217.60/month; Step 3: R40.60/kL = R267.96/month).

6.4 Payback Period

Estimated install cost: ~R3,000 (BOM only, excl. labour).

Payback at Step 1 tariff: 3000 / 160.51 = 18.7 months (~2 years)

Payback at Step 3 tariff: 3000 / 267.96 = 11.2 months (~1 year)

7. Commissioning Checklist

| # | Item | Req | Pass | Sign | Date |
| --- | --- | --- | --- | --- | --- |
| 1 | All cable glands PG9/PG7 IP68 sealed on enclosure | Yes | ☐ |  |  |
| 2 | Enclosure IP65 rating verified (no gaps, door sealed) | Yes | ☐ |  |  |
| 3 | Mornsun PSU output set to 12.0V ±0.2V (no load) | Yes | ☐ |  |  |
| 4 | LM2596 output set to 5.0V ±0.1V (no load) | Yes | ☐ |  |  |
| 5 | ESP32-C6 powers up, OLED shows firmware version | Yes | ☐ |  |  |
| 6 | WiFi connects to network, NTP syncs within 60s | Yes | ☐ |  |  |
| 7 | DS3231 RTC maintains time during 5-min WiFi outage | Yes | ☐ |  |  |
| 8 | A02YYUW reports distance (test in air: 20-30 cm typical) | Yes | ☐ |  |  |
| 9 | JoJo level % display on OLED matches calculated depth | Yes | ☐ |  |  |
| 10 | YF-S201 flow meter pulses when water flows (test tap) | Yes | ☐ |  |  |
| 11 | Cistern float switch toggles (simulate with manual press) | Yes | ☐ |  |  |
| 12 | Well pump relay energises/de-energises on command | Yes | ☐ |  |  |
| 13 | BDD pump activates on GPIO15 HIGH (verify no pump hunting) | Yes | ☐ |  |  |
| 14 | All 4 aviation LEDs flash per programmed pattern | Yes | ☐ |  |  |
| 15 | Toilet WS2812B shows correct colours | Yes | ☐ |  |  |
| 16 | Home Assistant discovers all 12 entities | Yes | ☐ |  |  |
| 17 | Schedule active 17:00-07:00 (test with time override) | Yes | ☐ |  |  |
| 18 | MCB-04 (2A DC) trips correctly on pump short-circuit | Yes | ☐ |  |  |
| 19 | Earth resistance <30Ω at earth spike | Yes | ☐ |  |  |
| 20 | No 220V on enclosure metalwork (megger test >1MΩ) | Yes | ☐ |  |  |
| 21 | DPSH: pipework pressure test (no leaks at joints) | Yes | ☐ |  |  |
| 22 | BDD pump primes and delivers water to cistern | Yes | ☐ |  |  |
| 23 | Cistern does not overfill (float switch de-energises pump) | Yes | ☐ |  |  |
| 24 | 24-hour soak test: no faults, no memory leaks | Yes | ☐ |  |  |
| 25 | Photographic record of completed installation | Yes | ☐ |  |  |

8. SANS 10142-1 Compliance Notes

The installation must comply with SANS 10142-1 (The Wiring of Premises). Key requirements:

All 220V AC wiring in 4mm² TPS in 20mm PVC conduit

Pool DB is the origin of supply for the Water PLC sub-DB

Dedicated 10A MCB (MCB-01) per SANS 10142-1 regulation 6.3

Earth spike ≤30Ω per SANS 10142-1 regulation 8.2.2

All accessible conductive parts bonded to earth

SANS 10142-1 requires that modifications to an existing DB be done by a licensed electrician

9. Future Upgrade — Solar and Battery

This section is reserved for future expansion. No solar/battery components are specified in v0.3.

Future upgrade path (not in scope for v0.3):

Solar panel: 12V 100W rigid (R1,500-2,000 at Builders Warehouse / Takealot)

Solar charge controller: MPPT 10A (R350-500, e.g. EPSolar/ET4415)

Battery: 12V 7Ah sealed lead-acid (R400-600, for RTC + overnight clock only)

Benefits: Grid independence for water system; estimated R60-80/month additional saving

Note: BDD pump requires 12V 2A continuous; solar+battery sizing requires professional assessment

10. Glossary

| Term | Definition |
| --- | --- |
| AO3400A | N-channel logic-level MOSFET, SOT-23 package, used as electronic switch for 12V pump |
| A02YYUW | Ultrasonic ranging module, IP67, UART interface, 3-450 cm range, used for JoJo tank level |
| BDD pump | BDD 12V DC centrifugal water pump, 800 L/h, 5m head, gravity-fed from JoJo tank bottom outlet |
| BOM | Bill of Materials — full list of components required for the build |
| DS3231 | Precision real-time clock module, I²C, ±2 ppm accuracy, with CR2032 battery backup |
| DPSH | Deep well submersible pump (existing on property) |
| ESPHome | Open-source firmware for ESP32/ESP8266 microcontrollers, YAML-based configuration |
| GPIO | General Purpose Input/Output — microcontroller pin |
| HA | Home Assistant — open-source home automation platform |
| HDPE | High-Density Polyethylene pipe, 15mm OD, PN12, for water mains |
| HL-52S | 5V DC relay module with opto-isolated input, 10A contacts, for well pump control |
| JoJo tank | Rotationally-moulded plastic water storage tank, typically 1000L capacity |
| LEDC | LED Control — ESP32 hardware PWM peripheral, used for aviation LED flashing |
| LM2596 | Step-down DC-DC buck converter IC, used to produce 5V from 12V rail |
| MQTT | Message Queuing Telemetry Transport — lightweight pub-sub protocol for HA integration |
| NTP | Network Time Protocol — used to synchronise ESP32 system clock |
| OLED | Organic Light-Emitting Diode display, 0.96" 128×64, I²C interface |
| PSU | Power Supply Unit |
| P&ID | Piping and Instrumentation Diagram — engineering drawing showing process flow |
| SANS | South African National Standard |
| TPS cable | Thermoplastic Sheathed cable, standard SA household wiring cable |
| UART | Universal Asynchronous Receiver-Transmitter — serial communication protocol for A02YYUW |
| WiFi 6 | IEEE 802.11ax — latest WiFi standard, used by ESP32-C6 |
| WS2812B | RGB LED with integrated controller, single-wire NeoPixel protocol, for toilet status indicator |
| YF-S201 | Hall-effect flow meter, 1/2" BSP, 1-30 L/min, outputs pulses proportional to flow rate |

11. ESPHome Firmware

The ESPHome YAML configuration is provided as a separate file: water-plc-v0.3.yaml

Platform: ESPHome 2024.x

Target: ESP32-C6 DevKitC-1

Framework: Arduino

Network: WiFi 6, MQTT/API to Home Assistant

Entities exposed to HA: 12 (see §4 for list)

OTA update: enabled with password protection

OTA port: 3232

11.1 Home Assistant Entities

| Entity | Type | GPIO / Source | Description |
| --- | --- | --- | --- |
| sensor.jojo_level_percent | sensor | GPIO20/21 A02YYUW | Tank level as percentage |
| sensor.jojo_level_liters | sensor | derived | Tank level in litres (0-1000) |
| sensor.water_flow_rate_lpm | sensor | GPIO3 YF-S201 | Flow rate in L/min |
| sensor.daily_water_liters | sensor | derived | Daily cumulative flow |
| binary_sensor.cistern_full | binary_sensor | GPIO2 | True when cistern float high |
| binary_sensor.cistern_demand | binary_sensor | derived | True when cistern needs refill |
| switch.jojo_pump_manual_override | switch | GPIO15 | Manual pump override |
| switch.well_pump_manual_override | switch | GPIO18 | Manual well pump override |
| binary_sensor.schedule_active | binary_sensor | derived | True between 17:00-07:00 |
| binary_sensor.system_fault | binary_sensor | derived | True on any fault |
| sensor.wifi_rssi | sensor | ESP32-C6 | WiFi signal strength |
| light.aviation_lights | light | GPIO10-13 | 4 aviation LEDs grouped |
| light.toilet_indicator | light | GPIO16 WS2812B | Toilet RGB status indicator |

12. Supplier Quick Reference

| Supplier | Website | Stock Lead Time | Notes |
| --- | --- | --- | --- |
| Communica | communica.co.za | Fast (2-5 days) | PSUs, ESP32, sensors, cable, MCBs |
| Rabtron | rabtron.co.za | Fast (Joburg stock) | LEDs, resistors, capacitors, discretes |
| Mantech | mantech.co.za | Fast | Industrial PSUs, terminal blocks |
| Micro Robotics | microrobotics.co.za | Fast | ESP32, sensors, breakout boards |
| FastenRight | fastenright.co.za | Fast | Fasteners, bezels, gland kits |
| Takealot / AliExpress | takealot.com | Slow (3-14 days) | Backup sourcing |

End of Document

This document is the complete technical specification for the Domestic Water Replenishment Controller v0.3, dated 2026-07-18. For build questions, refer to the ESPHome YAML file (water-plc-v0.3.yaml), the 4 external drawings (D-01 through D-04), and the design notes (DESIGN-NOTES-v0.3.md).